Circuit Analysis Lab #2

Post-Lab Question Answers

Name: Ayesha Naseem       
Registration No: 2025-CE-19

## 1. Define Ohm’s law.

Ohm’s law says that the voltage across a resistor is equal to the current through it multiplied by its resistance. In equation form, V = I × R. This relation helps us find any one quantity if the other two are known.

## 2. What is a resistor?

A resistor is a basic electronic component that limits the flow of electric current. It is used in circuits to control current, produce voltage drops, and protect other components from excessive current.

## 3. How does a resistor work?

A resistor opposes the movement of charge in a circuit. Because of this opposition, part of the supplied electrical energy is converted into heat, and a voltage drop appears across the resistor whenever current passes through it.

## 4. What is your inference from the input and output waveforms on the oscilloscope?

The waveforms remain sinusoidal because the circuits used in this lab contain only resistors. There is no phase shift in a purely resistive circuit, so the input and output waves rise and fall together. The main change is in amplitude, since each resistor gets only a portion of the applied voltage.

## 5. Show calculations of 7 V input signal (Figure 10) from signal generator.

For the 7 V peak-to-peak input, the peak value is Vp = 7/2 = 3.500 V and the RMS value is Vrms = 3.500/√2 = 2.475 V.

Voltage from oscilloscope divisions: if volt/div = 1 V and the waveform height is 7 divisions, then Vpp = 7 × 1 = 7 V.

Frequency and period from oscilloscope divisions: if time/div = 1 ms and one cycle covers 5 divisions, then T = 5 ms and f = 1/T = 200 Hz.

Current using Ohm’s law: total resistance of Circuit B is 13.1 kΩ, so Irms = Vrms / Rtotal = 2.475/13100 = 0.189 mA. Peak current can also be written as Ipeak = Vp / Rtotal = 3.500/13100 = 0.267 mA.