Circuit Analysis Lab #2

Explanation and Working

Name: Ayesha Naseem
Registration No: 2025-CE-19

# Objective

This experiment is about learning how to use the signal generator and oscilloscope for sinusoidal signals in simple resistive circuits. The main quantities studied in the lab are peak voltage, peak-to-peak voltage, RMS voltage, frequency, period, and current.

# Short discussion

In a series circuit made only of resistors, the waveform shape does not change. The output across each resistor is still a sine wave and it stays in phase with the input. The difference is that the amplitude becomes smaller because the source voltage is shared between the resistors according to voltage division.

For a sine wave, the main relations used in this lab are:

Vp = Vpp / 2

Vrms = Vp / √2

T = 1 / f

V = I × R

# Experiment 1: Circuit A

Given values: input = 5.0 Vpp sine wave, frequency = 100 Hz, R1 = 5.1 kΩ, R2 = 3.3 kΩ, and total resistance = 8.4 kΩ.

First, the input signal values were found. Since Vpp = 5.0 V, the peak value is Vp = 5.0/2 = 2.500 V. The RMS value becomes Vrms = 2.500/√2 = 1.768 V. The time period is T = 1/100 = 0.010 s, which is 10.0 ms.

The voltage across each resistor was found using voltage division. For R1, Vpp(R1) = 5 × (5.1/8.4) = 3.036 V. Therefore Vp(R1) = 1.518 V and Vrms(R1) = 1.073 V.

For R2, Vpp(R2) = 5 × (3.3/8.4) = 1.964 V. Therefore Vp(R2) = 0.982 V and Vrms(R2) = 0.694 V. The sum of the resistor drops is 5.000 Vpp, which matches the input signal.

## Result table for Circuit A

| Location | Peak Voltage (Vp) | RMS Voltage (Vrms) | Peak-to-Peak (Vpp) |
| --- | --- | --- | --- |
| Input signal | 2.500 | 1.768 | 5.000 |
| R1 (5.1 kΩ) | 1.518 | 1.073 | 3.036 |
| R2 (3.3 kΩ) | 0.982 | 0.694 | 1.964 |

# Experiment 2: Circuit B

Given values: input = 7.0 Vpp sine wave, frequency = 200 Hz, R1 = 4.7 kΩ, R2 = 3.3 kΩ, R3 = 5.1 kΩ, and total resistance = 13.1 kΩ.

For the source, Vp = 7.0/2 = 3.500 V and Vrms = 3.500/√2 = 2.475 V. The period is T = 1/200 = 0.005 s, which is 5.0 ms.

Using voltage division, the resistor voltages are: Vpp(R1) = 7 × (4.7/13.1) = 2.511 V, Vpp(R2) = 7 × (3.3/13.1) = 1.763 V, and Vpp(R3) = 7 × (5.1/13.1) = 2.725 V.

The combined voltages are also useful: Vpp(R1+R2) = 7 × (8.0/13.1) = 4.275 V and Vpp(R2+R3) = 7 × (8.4/13.1) = 4.489 V.

Using RMS values for the whole circuit, the current is Irms = Vrms / Rtotal = 2.475 / 13100 = 0.189 mA. The peak current is Ipeak = Vp / Rtotal = 3.500 / 13100 = 0.267 mA.

## Result table for Circuit B

| Location | Peak Voltage (Vp) | RMS Voltage (Vrms) | Peak-to-Peak (Vpp) |
| --- | --- | --- | --- |
| Input signal | 3.500 | 2.475 | 7.000 |
| R1 (4.7 kΩ) | 1.256 | 0.888 | 2.511 |
| R2 (3.3 kΩ) | 0.882 | 0.623 | 1.763 |
| R3 (5.1 kΩ) | 1.363 | 0.964 | 2.725 |
| R1 and R2 | 2.137 | 1.511 | 4.275 |
| R2 and R3 | 2.244 | 1.587 | 4.489 |

# How the 7 V input can be shown on the oscilloscope

One simple way is to take the vertical scale as 1 V/div and the horizontal scale as 1 ms/div. If the waveform height covers 7 vertical divisions, then the peak-to-peak value is 7 V. If one complete cycle covers 5 horizontal divisions, then T = 5 ms and the frequency becomes 1/0.005 = 200 Hz.

Final Circuit: