Circuit Analysis

Lab 6

Questions/Answers

Assignment

Submitted by:

Wardah Shahid

2025-CE-81

University of Engineering

and Technology

Questions/Answers

- Define Kirchhoff’s current law.

Ans:Kirchhoff’s Current Law states that:

The total current entering a junction (node) is equal to the total current leaving that junction.

Mathematical Form:

∑Iin​=∑Iout​

- Define Kirchhoff’s voltage law.

Ans:Kirchhoff’s Voltage Law states that:

The algebraic sum of all voltages around any closed loop in a circuit is equal to zero.

Mathematical Form:

∑V=0

3. What are the applications of regulated power supply?

Ans:A regulated power supply provides a constant and stable output voltage regardless of changes in input voltage or load conditions. It is widely used in the following areas:

1. Electronic Devices

Used in devices like computers, televisions, and radios to ensure stable operation and protect components from voltage fluctuations.

2. Communication Systems

Essential in communication equipment (transmitters, receivers) to maintain signal clarity and avoid distortion.

3. Laboratory Equipment

Used in labs for testing and experiments where precise and stable voltage is required.

4. Medical Equipment

Used in devices like ECG machines and other medical instruments that need reliable and constant power.

5. Industrial Systems

Used in control systems, automation, and machinery to ensure smooth and safe operation.

6. Battery Charging Circuits

Helps in safe and efficient charging by providing controlled voltage and current.

4.What are the limitations of KCL?

Ans: Kirchhoff’s Current Law has some limitations. It is not applicable at very high frequencies where displacement current becomes significant. It assumes that no charge is stored at the junction, which may not be true in circuits with capacitors. KCL also does not consider electromagnetic effects such as radiation. Moreover, it is mainly valid for lumped circuits and may not give accurate results in distributed systems.