Circuit Analysis

Lab 11

Questions/Answers

Assignment

Submitted by:

Wardah Shahid

2025-CE-81

University of Engineering

and Technology

Question/Answers

1. What is Norton's Theorem?

Norton’s Theorem states that any linear, bilateral circuit containing independent or dependent voltage and current sources and linear resistors can be simplified into an equivalent circuit. This equivalent circuit consists of a single constant current source (I_N) connected in parallel with a single equivalent internal resistance (R_N). The Norton current (I_N) is calculated by short-circuiting the load terminals, while the Norton resistance (R_N) is the equivalent resistance looking back into the open-circuited load terminals with all independent voltage sources replaced by short circuits and independent current sources replaced by open circuits.

2. What are the applications of Norton's Theorem?

Circuit Simplification: It reduces highly complex and dense electrical networks into a simple two-component parallel circuit, making calculation much easier.

Load Variation Analysis: It is highly useful in systems where the load resistance changes frequently (like power distribution grids), allowing quick recalculation of load current without re-solving the entire network.

Source Transformation: It provides the foundation for source transformation, enabling easy conversion between Thevenin's voltage-source equivalent and Norton's current-source equivalent circuits.

Power System Modeling: It is widely used in electronic design automation (EDA) tools and power system analysis to model complex networks as simple current injectors for fault analysis.

3. What are the theoretical and practical limitations of Norton's Theorem?

Non-Linear Elements: Theoretically, it cannot be applied to circuits containing non-linear components such as diodes, transistors, or varistors, as their resistance changes with voltage or current.

Unilateral Components: It does not work for circuits containing unilateral elements (like rectifiers) that change behavior based on the direction of current.

Magnetic Coupling: Circuits with non-linear magnetic coupling or induction cannot be easily simplified using standard Norton parameters.

Internal Power Losses: Practically, the equivalent circuit only predicts the behavior of the current and voltage at the load terminals; it does not accurately represent the internal power consumption or efficiency of the original network.

Component Tolerance and Ideal Impedance: In a practical laboratory setup, real current sources do not have infinite parallel resistance like ideal ones, and resistor tolerances introduce small deviations between calculated and measured values.